I Know You

Andrew McCleary
Quinnipiac University

College of Computing and Engineering
Hamden, CT
andrew.mccleary@quinnipiac.edu

Brady Galligan
Quinnipiac University

College of Computing and Engineering
Hamden, CT
brady.galligan@quinnnipiac.edu
Aislin Hayes 
Quinnipiac University

College of Computing and Engineering
Hamden, CT
ashayes@quinnpiac.edu
Jacob Levin
Quinnipiac University

College of Computing and Engineering
Hamden, CT
jacob.levin@quinnipiac.edu

Chetan Jaiswal
Quinnipiac University

Associate Chair – Computing

Chetan.Jaiswal@quinnipiac.edu

Soumyashree Sahoo

Quinnipiac University

Assistant Professor of Computer Science

Soumyashree.Sahoo@quinnipiac.edu

Aditya Gupta

Quinnipiac University

Part-Time Faculty

Aditya.Gupta@quinnipiac.edu

Abstract

The ability to quickly and accurately identify an individual can be vital in high-stake environments, including security, healthcare, education, and professional networking. This task poses a challenge for individuals managing memory or face recognition impairments, like Dementia or Prosopagnosia (Face Blindness). To bridge this gap, we developed the I Know You Glasses: a wearable device that combines AR (Augmented Reality) and AI (Artificial Intelligence) to provide real-time, non-obtrusive facial identification onto the user’s HUD. Our core innovation is a “hybrid” system architecture designed to solve the two biggest issues in comparable wearable tech: high processing lag and the difficulty of recognizing faces at awkward angles.

To achieve this, we moved away from the traditional self-contained model, which often suffers from stuttering video and high-power consumption. Instead, we engineered a division of labor between the glasses and a remote server. The glasses themselves handle the lightweight task of tracking faces in the field of vision of the user locally. Only when it detects a high-quality front-facing facial image do the glasses trigger to take a crop and send it over to the server for identification. This transfer ensures that the device stays responsive, and information on the HUD can remain up to date without the “lag” that usually plagues real-time wearables.

As a demo of the product, we collected publicly available scraped information on Connecticut residents. Our main source of photos of faces is from LinkedIn profile pictures obtained through Google Custom Search API. This data can be accessed through an app made with Unity and the HUD display in the glasses by simply looking at the corresponding person. The app, connected to the glasses, can customize the HUD and recall recently viewed people.

1 Introduction

Being able to put a name to a face is a fundamental component of effective human interaction and relationships, yet this capability is increasingly challenging in our modern, fast-paced, and security-conscious world. This challenge manifests critically across three distinct user domains: security and surveillance, where immediate identification is essential for threat mitigation; professional networking, where recalling a contact’s name is crucial for relationship building; and for individuals managing mental conditions (such as dementia or prosopagnosia) that impair memory and therefore face recognition. In each scenario, the failure to identify or recall a face can have significant professional, personal, or safety consequences.

Over the last decade, AI-powered facial recognition has transitioned from niche government use to pervasive consumer technology, creating a global debate about its utility and ethics. From state-level surveillance to biometric device authentication like Apple Face ID [4], the technology’s presence is undeniable. Just recently, the rise of reverse image search engines, such as PimEyes [1] and ProFaceFinder [2], has highlighted both the power and the inherent privacy risks associated with vast, web-scraped databases of faces. This dual-use nature highlights the immense potential for both security and convenience yet also raises concerns about the high potential for surveillance and misuse, underscoring the current state of the field. Key challenges currently facing the industry include concerns over algorithmic bias (especially impacting women and people of color), unauthorized surveillance, and patchwork regulatory compliance across jurisdictions.

Despite rapid advances, current facial recognition solutions in the wearable AR space are insufficient for real-world, dynamic use by the specialized user, primarily due to the architectural and ethical flaws. Existing experimental systems are financially unsustainable for general use and are vulnerable to instability due to their dependence on costly, centralized 3rd-party APIs. Solutions relying on services like PimEyes and ProFaceFinder operate with non-transparent data sourcing, creating significant legal and ethical risks for developers and users. Many commercial smart glasses prioritize non-recognition features, leaving facial recognition as an undeveloped or under-optimized function, which increases the likelihood of inaccurate recognition due to insufficient image variety and scale.

To address the critical gaps of high latency, unsustainable cost, and reliance on unstable 3rd-party APIs, our team has designed the I Know You Glasses. The I Know You glasses are an AR-driven, AI-powered wearable that provides real-time, unobtrusive identification by fundamentally changing the system architecture.

We achieved this by eliminating the 3rd-party API dependence through creating our own web-scraped ‘face bank’ and running our own proprietary facial identification software, as well as self-hosting all of this data on an AES-128 encrypted server. Furthermore, the system processes data and outputs the information directly to the device’s built-in AR HUD on a much quicker time frame than existing mobile-app-related solutions, minimizing the critical problem of latency. This paper details the design, implementation, and assessment of the I Know You Glasses system, presenting a robust model for developing an independent facial recognition solution, specifically focusing on a hybrid use of MediaPipe and DeepFace’s Facenet512 to overcome the challenge of pose variation. We detail more rigorously later in this paper for evaluating our approach's performance and present the results demonstrating its significant speed and cost advantages over current paradigms, ultimately seeking to demonstrate a viable, self-contained architecture for discreet, real-time facial recognition in a wearable format.

2 Literature Review

PimEyes is an online facial recognition software and API that allows users to upload a photo and search for matching appearances [1]. It uses AI powered detection and an extensive face database built through web-scraping, returning URLs where that person’s face appears. The company markets the service as a way for individuals to monitor and manage their online presence. Its primary advantage is speed and accuracy, with the platform often praised for how quickly and precisely it identifies matches. It is also framed as a privacy tool for users seeking to control their digital footprint. However, PimEyes has been heavily criticized for misuse, including concerns that third parties could use it for surveillance, stalking, or other unethical purposes. Legal disputes in Europe have also placed the platform under scrutiny, with lawsuits questioning whether its data scraping practices comply with privacy laws. This tension between personal security and privacy risks highlights the broader ethical dilemma surrounding facial recognition technology.

ProFaceFinder is a reverse image search engine that scans online platforms and social media for faces matching a user-uploaded photo [2]. It primarily markets itself for anti-scam and anti-catfishing purposes, offering AI image detection and geolocation tools to help verify identities. Strengths of this project include a clear, specific use case and a user-friendly design that makes the tool approachable. Its design also avoids direct API access, reducing potential misuse compared to PimEyes. However, ProFaceFinder paywalls its search results, preventing users from verifying their accuracy. Additionally, it claims about scarping social media raise legal and ethical concerns. Transparency issues around data sourcing and legality remain significant weaknesses in the project’s credibility.

Face SDK by Regula is a biometric verification toolkit designed to support facial recognition, liveness detection, and image quality analysis across multiple platforms [3]. It is designed for high-security applications such as border control, banking, and identify verification, featuring active and passive liveness detection. Its biggest strengths include flexibility and adaptability, with applications across industries such as border control, finance, and law enforcement. The software complies with presentation attack detection (PAD) standards and can be customized to fit various institutional needs, making it appealing for organizations requiring reliable biometric verification. Another advantage is its support for multi-platform use, which broadens its utility across both hardware and software systems. However, its reliance on high-quality images presents limitations, such as occlusions, poor lighting, or low-resolution inputs can reduce accuracy. Privacy and data protection remain significant concerns, as facial data is highly sensitive and prone to misuse if not managed responsibly. Additionally, its cost and integration complexity may pose barriers for smaller businesses or organizations. Despite these weaknesses, Face SDK represents one of the more robust and adaptable biometric solutions available.

Apple Face ID is Apple’s proprietary biometric authentication system integrated into iPhones and iPads [4]. It uses infrared mapping to generate a detailed scan of a user’s face and stores the encrypted data locally in the device’s Secure Enclave. The system adapts over time, recognizing gradual changes in a user’s appearance, and is designed to unlock devices, authorize payments, and log into applications. Strengths of Face ID include its strong anti-spoofing measures, ease of use, and its security model, which avoids reliance on cloud-based facial data storage. Apple has also built in accessibility features, such as adapting to users wearing glasses or facial coverings. However, Face ID remains limited to Apple’s ecosystem, restricting its reach to those who own Apple products. Broader ethical concerns also exist around biometric dependence, even though Apple’s approach to on-device storage mitigates some of these risks. Overall, Face ID exemplifies a consumer-facing facial recognition tool that balances security and convenience while avoiding some pitfalls of broader recognition systems.

Vive Eagle Smart Glasses are a pair of AR-powered glasses that provide real-time translation, AI-assisted search, and a built-in camera and speaker system [5]. They showcase potential facial recognition applications, though this feature has not been fully developed. Their biggest strengths are multifunctionality, a modern chipset, and compatibility with both Android and iOS devices, which make them attractive to a wider user base. In practice, however, weaknesses emerge, including limited geographic availability and an awkward, bulky design that reduces comfort for long-term use. Additionally, the voice-based input system requires speaking commands aloud, which can feel impractical in public or professional environments. These limitations raise questions about how smoothly glasses integrate into everyday life. While they represent a strong step toward consumer AR devices, they are not yet optimized for seamless or discreet use.

Vital Smart Glasses are marketed as a lightweight and affordable option for everyday use, offering features such as translation across 130 languages, Bluetooth pairing, hands-free calling, music streaming, and voice note recording [6]. They emphasize accessibility and convenience, aiming to keep users connected without constant phone use. The main advantage of this product lies in its comfort-first design and affordability, making it more accessible compared to high-end smart glasses. Its range of practical features also positions it as a tool for casual convenience, appealing to users who want wearable connectivity without excessive cost. However, the weaknesses of the product include vague technical details in its marketing materials, raising doubts about how well its features perform in practice. Additionally, translation accuracy and performance may not meet the needs of professional or high-stakes contexts, limiting its scope to novelty or light use. These weaknesses suggest that while Vital Smart Glasses provide a budget-friendly entry point to AR-style wearables, they lack the robustness and reliability needed for more advanced applications.

Viture Luma Smart Glasses are marketed as premium AR/VR glasses that prioritize high-resolution displays, integrated cameras and microphones, and advanced motion tracking capabilities [7]. The biggest strengths of the Luma glasses include their visual quality and positional tracking, which enhance entertainment and AR applications. They also support hand-tracking features, offering more interactive possibilities than lower-end competitors. However, several weaknesses limit their appeal, including uncertainty about real-world performance and usability in everyday scenarios. Their higher cost also makes them less accessible to casual consumers. Additionally, while advanced features sound appealing, they may introduce complexity or trade-offs that affect ease of use. As a result, these glasses suggest an ambitious push toward immersive AR and VR integration but remain niche in terms of mainstream adoption.

I-XRAY was a student-led project at Harvard University that used Meta’s Ray-Ban smart glasses combined with PimEyes, scrapers, public databases, and ChatGPT to identify people and provide summarized information about them [8]. The project was framed as a privacy awareness campaign rather than a commercial product, aiming to spark discussion about surveillance and data ethics. Its strengths include the scope of databases it could draw from, giving it potentially broad recognition capabilities. However, weaknesses were significant, including slow performance caused by multiple API calls, reliance on consistent database upkeep, and limitations in how data was displayed, since it only appeared through a mobile app rather than directly in the glasses. Additionally, no public updates have been reported since early 2024, suggesting limited continuity. Despite these flaws, I-XRAY remains one of the closest real-world parallels to integrating facial recognition and AR glasses, raising valuable questions about the future of such technologies.

AI-Based Facial Recognition Smart Glasses for the Visually Impaired were created by students at SECAB to assist people with vision impairment [9]. The glasses use a camera to detect objects and people, calculate distances, and allow the user to build a personal database of known faces. This adaptability is one of the system’s major strengths, as users can register new individuals on the fly, improving recognition over time. The device is also relatively low cost, making it more feasible for accessibility-focused applications. However, the system also faces limitations, particularly when the database lacks sufficient variation in stored images, leading to inaccurate recognition. Poor-quality input images also reduce reliability. While promising, the project demonstrates both the potential and the challenges of tailoring facial recognition technology for accessibility, highlighting the difficulty of relying on small, user-built databases.

AI-Powered Glasses for the Visually Impaired, developed by students at Parul University, expand on the SECAB design with added functionality such as image-to-text conversion using Google’s Tesseract OCR. This feature allows the glasses not only to detect objects and faces but also to read text aloud, making them more useful in real-world applications [10]. Strengths of this system include improved recognition accuracy and expanded features, which increase its value for visually impaired users. However, weaknesses are clear, including higher production costs and slower processing speed due to reliance on a Raspberry Pi. Additionally, the number of detectable objects remains limited, restricting versatility in certain contexts. These weaknesses show that while the project has advanced beyond earlier models, it still faces technical and economic barriers to widespread adoption. Nevertheless, it illustrates how AI glasses can evolve to better serve specialized populations, blending recognition with assistive functions.

Android VR Development provides the base operating system for many AR and VR platforms, including devices like Pico and Meta headsets [11]. Its major strength is adaptability, as Android is widely known and accessible, making it easier for developers to create prototypes and cross-device applications. The system also benefits from its open ecosystem, which encourages experimentation and lowers the barrier to entry. However, fragmentation across devices presents a challenge, as different hardware variations can complicate development and reduce consistency. Compared to proprietary systems, Android may also lack some optimizations or deep integrations. Despite these challenges, Android serves as a versatile backbone for XR development, especially in early-stage innovation.

Unity VR Development offers developers a set of drag-and-drop XR tools and UI systems through its XR Toolkit, which simplifies the process of building immersive applications. Its strengths include comprehensive documentation, pre-built templates, and a user-friendly interface that accelerates prototyping [12]. Unity also supports a large developer community, providing resources and support for newcomers. However, its weaknesses include performance limitations compared to custom-built engines, particularly for highly demanding applications. Another concern is the reliance on Unity’s ecosystem, which may limit flexibility in the long run. Still, Unity remains one of the most accessible and powerful platforms for both beginners and professionals entering the VR/AR development space.

The Pico 4 SDK integrates with Unity to enable developers to compile applications directly for Pico headsets, while also providing templates for features like passthrough, hand tracking, and device-specific optimizations. Its greatest advantage is ease of deployment, which streamlines prototyping and allows quick hardware testing [13]. The SDK also supports strong hardware integration, making it valuable for developers targeting Pico’s ecosystem. On the downside, its utility is largely limited to Pico devices, reducing cross-platform applicability. Its reliance on Unity also narrows its usefulness outside that framework. Despite these constraints, the Pico 4 SDK represents a practical development tool for building and testing AR/VR applications, especially in early-stage design and experimentation.

The Viture Luma SDK is a set of developer tools published officially by the Viture team to make development on their hardware as straightforward as possible. This kit provides libraries for separating out displays from a mobile device and the glasses, as well as more streamlined access to the hardware onboard the glasses, such as pre-written tracking algorithms for the cameras. This SDK also provides substantial documentation and assistance for development [13] showing important methods and providing samples on how to use them.

3  Our Approach

For our facial recognition software, we employ a hybrid model detection-identification pipeline utilizing MediaPipe’s face mesh model in tandem with DeepFace’s Facenet512 model. We utilize the MediaPipe model to do lightweight face detection, capture and analysis client side, while the DeepFace model is used for creating the face embeddings for comparison/identification server side. The face mesh model was primarily selected for its tracking feature, which is useful for capturing several crops of the same face detection and for following multiple people in the same frame. Additionally, the geometric landmarks vector which is output by all MediaPipe models is very useful for performing rigorous pose analysis, allowing for the capture of only front-facing crops. The Facenet512 model was primarily selected for its extremely detailed 512-dimension face encoding vectors, which have been found to be the most accurate model for identifying distinct people [14]. We chose to split our models across a client-server architecture to split the computational workload, finding that when both models were running on the same device, performance dropped significantly.

As is the case with all existing publicly available facial recognition models, we ran into numerous issues attempting to mitigate the problem of pose variation—our models would find a higher similarity between two different people than between the same person at extreme angles. After extensive attempts to eliminate this issue through careful recognition threshold adjustments based on a pose variation scoring metric, we determined that there are simply some angles that recognition should not be attempted on, and that it was ideal to only capture front-facing faces for storage and future recognition. This concluded in the creation of three separate identification cases: Ignore, ID, and Capture. For the Ignore case, which is triggered when a pose does not meet even the baseline pose score threshold for identification, we simply ignore the detection and do not attempt recognition. For the ID case, which is triggered when a pose meets the baseline pose score threshold for identification but not the stricter threshold for capturing a new face, identification against existing embeddings already stored in the database is attempted, but should it fail, the face cannot be captured to create a new identity. For the Capture case, which is triggered only when the strict pose score threshold for capturing a new face is met, identification is still tried initially, but if it is unsuccessful, the face detected is likely to be an unidentified person and is captured for future recognition.

In order to capture the faces seen by the glasses’ camera, we determined that it was latency inefficient to send the entire video stream continuously, and furthermore that it was computationally inefficient to attempt to identify every single frame of a detection. We opted instead to run the MediaPipe face mesh locally on the AR glasses, using a Raspberry Pi for processing, to only send the face crops to the DeepFace recognition software running on the server, allowing local utilization of the face mesh tracking for UI without any added latency. The application sends one crop for re-identification against the last five recently seen identities, and if it fails, it attempts to capture the face by sending ten crops for a more detailed check against all faces in the database, at which point if there is no match it will capture the new face and return the new id associated with it.

As previously mentioned, we used a Raspberry Pi to run our client Python program on the glasses’ camera stream. To power and secure the Pi, we designed and 3D-printed a custom battery pack and neckband to allow for portable access. Attached by several cables consisting of MicroHDMI to HDMI and a data/power line over USB in to convert to DisplayPort over USB-C, the glasses acted as an AR screen for the program’s UI, which was tuned to display only the boxes and obtained information of a successful match over a live visual feed.

For database access, we deployed a PostgreSQL database on a remote server which we accessed through a Python RESTful API. For asynchronous method calls, we used the asyncio module. We utilized the popular third-party PostgresSQL pgvector extension in our database for the ability to store the face embeddings directly. In our custom API, we employ FAISS or the Facebook Artificial Intelligence Similarity Search, for automated indexing of the large vector database based on cosine similarity for the quick retrieval of matches.

For gathering data for the entries to populate the database, we utilized the Python framework, Scrapy. We wrote code that mimicked the POST request the API voteref.com uses to call its data searches. This yielded names, ages, and home addresses of Connecticut residents. Then using n8n, an opensource workflow automation platform, we scraped LinkedIn also aimed at Connecticut residents.

4  Our Contribution

Our primary contributions to face recognition software are in our detection and identification logic and overall system architecture. These contributions are a pose-variation fix for real time facial recognition technology, a case-based reidentification system for portable face recognition on several people at once, and a tracking and identity lock class for real-time AR face tracking allowing for a lighter weight client.

Inspired by other production face recognition software on the market, we decided to use the geometric vectors output by face mesh to determine how front facing a person’s head is before attempting to identify them. We also had a simple function to determine the sharpness of the crop. Only if the detection passed both of these tests could the client attempt to identify the person’s face, rejecting all those that were not high enough quality and waiting for a better angle. This effectively solved the pose variation problem for real time facial recognition.

Our identification logic is split into two cases: ID case and Re-ID case. In the ID case, if there are no recent identifications, our client collects 10 high-quality crops of a new face, then sends them to the server where they are processed by Facenet512, averaged together and normalized, and then checked against the encodings stored within our database. Otherwise, in the Re-ID case, if there have been recent identifications, the last five most recently seen IDs and one captured high-quality crop are sent to be checked for a high-similarity match on the server. If this limited check fails, a full check is run against the single crop, and if that fails, the client will capture 10 crops for a full database search. This case-based logic allowed for lightweight and quick identification of several people within the same frame, minimizing processing lag and identification latency. These cases allowed for our implementation of a portable AR client and face recognition server architecture to work on numerous people in frame at once.

Early on in development, we were sending an entire continuous stream of crops from each detection on screen to the server for processing, displaying the results in real time as they flowed back to the client. This was highly inefficient, especially since people’s identities do not change between frames, assuming the program has knowledge that it is tracking the exact same face. MediaPipe’s face mesh model has powerful face tracking built in, but the internal box ID’s and direct box object references are not available through the high-level API. To circumvent this issue, we developed a Kalman filter-based temporal intersection over union box tracking class, and tied identification and UI information to that single track object, displaying it continuously without the need for reidentification. To do so, we implemented an identity locking system, where if the server returned either a high confidence (>0.65) identification or two consecutive identical results, the track would lock onto that person’s identity, preventing further attempts at reidentification. This allowed for the simultaneous identification and display of multiple people, increased accuracy regardless of a person’s current pose, and greatly reduced processing and latency.

5  Limitations

As touched upon in the introduction, facial recognition has many limitations: racial biases, the inability to distinguish between similar looking people (i.e. twins), and faces at extreme angles, to name some. Lighting is also a key factor as to whether DeepFace can recognize a face. Further, the face capture had an issue with the range where it would cutoff at an inconsistent distance away from the camera. While we attempted to remedy some of the problems, there is no catch-all solution to any of them.

The purpose of this project was to gain experience with areas we were unfamiliar with: AI face detection and recognition, scraping, and AR displays. Our end result certainly has some shortcomings, as it is limited by the current state of facial recognition and potentially inadequate quality of the collection of data. However, with the time and budget allotted to work on this project, we consider this end result to be adequate. Frequent testing shows the recognition is very consistent and accurate. As facial recognition evolves, so will our product.

We currently have no intention or interest in turning this into a commercial product. This product would introduce a great deal of ethical concerns. For instance, if the glasses were stolen in their current state, it would give access to an unauthorized individual the identities and personal data of the current database attached to the glasses. Having the ability to link a name to a face introduces many ethical concerns alone. Not to mention the ethical concerns of each individual use case as well as hosting the data of thousands of individuals. If we were to sell it, it would be without a pre-built database and only sold to trusted companies that fall within our use cases while recommending heavy security measures around the product.

6  Conclusion

As touched upon in the introduction, facial recognition has many limitations: racial biases, the inability to always distinguish between similar looking people (i.e. twins), and faces at extreme angles, to name some. Lighting is also a key factor as to whether DeepFace can recognize a face. Further, the face capture had an issue with the range where it would cutoff at an inconsistent distance away from the camera. While we were able to remedy some of these problems, there is no catch-all solution that we were able to come up with.

The purpose of this project was to gain experience with areas we were unfamiliar with: AI face detection and recognition, scraping, and AR displays. Our result has some shortcomings, as it is limited by the current state of facial recognition models and the potentially inadequate quality of web-scraped data. However, with the time and budget allocated to work on this project, we consider our end result and contributions to be adequate.

Frequent testing shows the recognition is very consistent and accurate. As facial recognition technology evolves, so would our project. That being said, we currently have no intention or interest in turning this into a commercial product, as this product would introduce numerous ethical concerns were it to be widely available. For instance, if the glasses were stolen in their current state, it would give access to an unauthorized individual the identities and personal data of the current database attached to the glasses. Having the ability to link a name to a face can be powerful and dangerous, especially when employing web-scraping tactics that allow for the potential identification of strangers. Not to mention the specific ethical concerns of each potential use case, as well as hosting the data of thousands of individuals. If we were to sell it, it would be without a pre-built database and only sold to trusted companies that fall within our use cases while recommending heavy security measures around the product.

REFERENCES

[1] S. N. Ilkal, S. S. Munshi, S. Katarki, N. Kotwal, M. Chikkond, and A. Makandar, “AI Based Facial Recognition Smart Glass for Visually Impaired Person,” Saudi Journal of Engineering and Technology, vol. 10, no. 06, pp. 270–276, Jun. 2025, doi: https://doi.org/10.36348/sjet.2025.v10i06.003.

[2] “Try the PimEyes face recognition search engine and find yourself on the Internet • PimEyes,” pimeyes.com. https://pimeyes.com/en

[3] “ProFaceFinder,” ProFaceFinder, Nov. 24, 2025. https://profacefinder.com/ (accessed Dec. 07, 2025).

[4] “Identity Verification Solutions & Forensic Devices by Regula,” Regula, 2024. https://regulaforensics.com/

[5] Apple, “About Face ID advanced technology,” Apple Support, Aug. 22, 2023. https://support.apple.com/en-us/102381

[6] “VIVE Eagle - New AI Glasses with Translation, Bluetooth, Music,” Vive.com, 2020. https://www.vive.com/us/product/vive-eagle/overview/ (accessed Dec. 06, 2025).

[7] E. Levine, “Vital Smart Glasses: Translate, Record & Connect Hands-Free,” VITAL SMART GLASSES, Aug. 06, 2025. https://www.vital-smartglasses.com/en/ (accessed Dec. 07, 2025).

[8] Harvard Law School, “I-XRAY: the AI glasses that reveal anyone’s personal details | Library Innovation Lab,” Harvard.edu, Jan. 29, 2025. https://lil.law.harvard.edu/events/i-xray-lunch/

[9] N. Satani, S. Patel, and S. Patel, “AI Powered Glasses for Visually Impaired Person,” Regular, vol. 9, no. 2, pp. 416–421, Jul. 2020, doi: https://doi.org/10.35940/ijrte.b3565.079220.

[10] Unity, “XR Interaction Toolkit | XR Interaction Toolkit | 3.0.1,” docs.unity3d.com. https://docs.unity3d.com/Packages/com.unity.xr.interaction.toolkit@3.0/manual/index.html

[11] Google, “Quickstart for Google VR SDK for Android,” Google for Developers, 2019. https://developers.google.com/vr/develop/android/get-started (accessed Dec. 06, 2025).

[12] Pico Developer, “PICO Developer,” PICO Developer, 2015. https://developer.picoxr.com/document/ (accessed Dec. 06, 2025).

[13] VITURE, “VITURE Unity XR SDK Documentation” [Online]. Available: https://developer.viture.com/unity/viture_unity_xr_sdk_doc (Accessed: Dec 06, 2025)

[14] “Benchmarks · serengil/deepface,” GitHub, 2026. https://github.com/serengil/deepface/tree/master/benchmarks (accessed Apr. 23, 2026).